# ADR510 R_BIAS Calculation

This document calculates the bias resistor (R_BIAS) for the ADR510 shunt voltage reference to provide a stable 1.0V to LMC6484 Pin 10 (Op-amp D non-inverting input). The AD5761 addressing is negligible.

## Parameters

- ADR510 Output Voltage (V_OUT): 1.0V

- Load Current (I_L): Negligible (<1nA, LMC6484 Pin 10)

- Shunt Current (I_Q): 100µA to 10mA (datasheet)

- Supply Voltage (V_S): +5V or +3.3V

- R_BIAS Formula:

RBIAS=Vs-VoutIL+IQ​​

- Power Dissipation:

P=(Vs-Vout)2/RBIAS

## Calculations

### For V_S = +5V

- R_BIAS = 10kΩ:

  - IQ=(5-1)/10000=0.4mA=400μA

  - Power: ( P = 4^2 / 10000 = 1.6mW )

- Previous (4kΩ):

  - ( IQ=(5-1)/4000=1mA), P = 4mW

- Reduction: 4mW to 1.6mW (60%).

## Recommendation

- Use R_BIAS = 10kΩ (1% tolerance, e.g., 10.0kΩ):

  - V_S = +5V: I_Q = 400µA, P = 1.6mW, stable.

  - V_S = +3.3V: I_Q = 230µA, P = 0.529mW, stable.

- Preferred: V_S = +5V (matches AD5761 VDD), R_BIAS = 10kΩ.

- Setup:

  - Connect ADR510 V+ (Pin 2) to LMC6484 Pin 10 and R_BIAS.

  - Connect R_BIAS to V_S (+5V).

  - Connect ADR510 V− (Pin 1) to ground.

  - Optional: Add 1µF capacitor across V+ to V− for noise filtering.

- Verify: Measure 1.0V ±3.5mV at LMC6484 Pin 10.